WMF58 Series

Glass Package Type NTC Thermistors

玻璃封装型负温度系数热敏电阻器

- For Temperature Sensing用于温度探测及控制

■ GENERAL 概述

WMF58系列负温度热敏电阻器体积小、热感应快、灵敏度高、能在高温和高湿等恶劣环境下长时间稳定工作。

主要应用于电磁炉、电压力锅、电饭煲、电烤箱、消毒柜、微波炉等家用电器的温度控制及温度探测；工业、医疗、环保、气象、食品加工设备的温度控制和探测。

WMF58 series negative temperature thermistors have small volume, fast thermal induction, high sensitivity, and can work stably in high temperature and high humidity for a long time.

It is mainly used for temperature control and detection of home appliances such as induction cooker, pressure cooker, rice cooker, electric oven, sterilizer, microwave oven, etc.

■ PART NUMBERING 产品编号

| WMF58 | A | - | 103 | F | 3950 | G |  | B | - | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

⑴     ⑵        ⑶   ⑷    ⑸    ⑹     ⑺       ⑻

⑴ Series系列：WMF58系列

⑵ Eductor typel: A-Axial lead heterotopy, B-Same End of radial lead

引出端类型：A-轴向引线异端 B-径向引线同端

⑶ Rated Zero Power Resistance & Tol. 额定零功率电阻及允差：103-10KΩ   502-5KΩ  2.5K – 2.5KΩ

⑷ Tolerance: 允差范围：F-±1%  G-±2%  H-±3%   J-±5%  K-±10%

⑸ B value B值：3,100 - 3,100ºK  4,050 - 4,050ºK

⑹ B value tolerance B值允差： F-±1%  G-±2%  H-±3%

⑺ Packing Type包装形式：B - Bulk 散装Special Code (RoHS) 特别代码：

⑻ R-Compliance with RoHS ( e.g.: Pb Free ) 符合RoHS要求 ( 如: 不含铅 )

WMF58A

WMF58B

| 代号 | D | L1 | L2±5 | d±0.05 |
| --- | --- | --- | --- | --- |
| B1 | 1.3±0.2 | Max 3.2 | 65 | 0.25 |
| B2 | 1.8±0.2 | 3.4±0.5 | 65 | 0.25 |

■ SPECIFICATION S TABLE规格表

| 序号 No. | 产品编号 Part No | 额定零功率电阻(R25) Rated Zero Power Resistance |  | B值(25℃/50℃) B Value(K) |  | 热耗散系数δ (mw/℃) Thermal Dissipation Constant | 额定功率 （mw） | 热时间常数τ(s) Thermal Time Constant | 工作温度(℃) Operating Temperature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 电阻值 （KΩ） | 允差 （±%） | 标称值 （K） | 允差 （±%） |  |  |  |  |
| 1 | WMF58A-202¨3839¨ | 2K | 1,2,3,5 | 3839 | 1,2,3 | ≥2.0 | ≤50 | ≤12 | -40-+250 |
| 2 | WMF58A-502¨3470¨ | 5K |  | 3470 |  |  |  |  |  |
| 3 | WMF58A-502¨3950¨ | 5K |  | 3950 |  |  |  |  |  |
| 4 | WMF58A-103¨3380¨ | 10K |  | 3380 |  |  |  |  |  |
| 5 | WMF58A-103¨3470¨ | 10K |  | 3470 |  |  |  |  |  |
| 6 | WMF58A-103¨3600¨ | 10K |  | 3600 |  |  |  |  |  |
| 7 | WMF58A-103¨3700¨ | 10K |  | 3700 |  |  |  |  |  |
| 8 | WMF58A-103¨3950¨ | 10K |  | 3950 |  |  |  |  |  |
| 9 | WMF58A-103¨3975¨ | 10K |  | 3975 |  |  |  |  |  |
| 10 | WMF58A-103¨4100¨ | 10K |  | 4100 |  |  |  |  |  |
| 11 | WMF58A-203¨3950¨ | 20K |  | 3950 |  |  |  |  |  |
| 12 | WMF58A-303¨3950¨ | 30K |  | 3950 |  |  |  |  |  |
| 13 | WMF58A-233¨4200¨ | 23K |  | 4200 |  |  |  |  |  |
| 14 | WMF58A-473¨3950¨ | 47K |  | 3950 |  |  |  |  |  |
| 15 | WMF58A-503¨3950¨ | 50K |  | 3950 |  |  |  |  |  |
| 16 | WMF58A-503¨4050¨ | 50K |  | 4050 |  |  |  |  |  |
| 17 | WMF58A-104¨3899¨ | 100K |  | 3899 |  |  |  |  |  |
| 18 | WMF58A-104¨3930¨ | 100K |  | 3930 |  |  |  |  |  |
| 19 | WMF58A-104¨3950¨ | 100K |  | 3950 |  |  |  |  |  |
| 20 | WMF58A-104¨3990¨ | 100K |  | 3990 |  |  |  |  |  |
| 21 | WMF58A-104¨4200¨ | 100K |  | 4200 |  |  |  |  |  |
| 22 | WMF58A-204¨3899¨ | 200K |  | 3899 |  |  |  |  |  |
| 23 | WMF58A-204¨4200¨ | 200K |  | 4200 |  |  |  |  |  |
| 24 | WMF58A-204¨4260¨ | 200K |  | 4260 |  |  |  |  |  |
| 25 | WMF58A-504¨4260¨ | 500K |  | 4260 |  |  |  |  |  |
| 26 | WMF58A-504¨4050¨ | 500K |  | 4050 |  |  |  |  |  |
| 27 | WMF58B-103¨3950¨ | 10K | 1,2,3,5 | 3950 | 1,2,3 | ≥2.0 | ≤3.5 | ≤6.5 | -40-+300 |
| 28 | WMF58B-103¨3435¨ | 10K |  | 3435 |  |  |  |  |  |
| 29 | WMF58B-503¨3950¨ | 50K |  | 3950 |  |  |  |  |  |
| 30 | WMF58B-503¨4036¨ | 50K |  | 4036 |  |  |  |  |  |
| 31 | WMF58B-104¨3950¨ | 100K |  | 3950 |  |  |  |  |  |
| 32 | WMF58B-104¨4200¨ | 100K |  | 4200 |  |  |  |  |  |
| 33 | WMF58B-204¨3899¨ | 200K |  | 3899 |  |  |  |  |  |

1-□代表阻值公差代号  2-□代表B值公差代号