WMZY Series

Smart Composite Thermistor-Varistor Device

智能复合型热、压敏电阻器

- For smart protection of overvoltage, overcurrent and overheat

用于过压、过流、过热智能保护

■ GENERAL 概述

伟林-中昊WMZY系列智能复合型热、压敏电阻器是一种智能分压保护元器件，可防止用错电源、雷击、静电放电、设备接触电源和交流线路感应干扰。主要用于智能电能表、机顶盒、VoIP网卡、无线电话、安全系统、传真机、主配线板（MDF）、数据调制解调器、模组模拟和ISDN线卡等过压、过流、过热保护。

热压敏电阻器可以作为过压、过流、过热保护元器件。作为过流、过热保护元器件时,其工作原理与热敏电阻器一样;作为过压保护元器件时，其工作原理简述如下：

当负载电压大于压敏电压时，压敏电阻产生隧道效应导通发热进入低阻态，同时将热量传给耦合（并联）在一起的热敏电阻，使热敏电阻温度上升。同时，压敏电阻导通后导致流过热敏电阻的电流增大，所产生的焦尔热令热敏电阻温度上升速度加快进入高阻态，从而产生动作起到如下保护作用：

1）当电压超过动作电压时，压敏电阻动作同时拑制电压；

2）动作以后的热、压敏电阻器（热敏电阻成高阻态）的功耗与负载分压：

2-1）当热、压敏电阻器功耗远远大于负载功率时，拑制电压保护；

2-2）当热、压敏电阻器功耗与负载功率相等或相近时，分压保护，平衡时压敏电阻关闭不导通；

2-3）当热、压敏电阻器功耗远远小于负载功耗时，分压保护，承受大部分电压，平衡时压敏电阻关闭不导通。

WMZY series is a kind of intelligent partial voltage protection composite Thermistor-Varistor Device, it can prevent the interference caused by wrong power supply, lightning, electrostatic discharge, equipment contact with power supply and AC line induction. WMZY series is mainly used for over-voltage, over-current and over-heat protection of smart meter, set-top box, VoIP network card, wireless telephone, security system, fax machine, main distributing frame (MDF), data modem, module simulation and ISDN line card.

Thermistor, varistor can be used as over-voltage, over-current, over-heat protection components.

As an over-current and over-heat protection component, its working principle is the same as thermistor; as an over-voltage protection component, its working principle is briefly described as follows:

When the load voltage is greater than the varistor voltage, the varistor will produce tunnel effect and enter the low resistance state, at the same time, the heat will be transferred to the coupled (in parallel) thermistor so that the temperature of the thermistor will rise. At the same time, when the varistor is turned on, the current flowing through the thermistor increases, and the resulting Joule heat accelerates the temperature rise of the thermistor into a high resistance state, thus the operation has the following protection functions:

- 1) When the voltage exceeds the operating voltage, the varistor works and controls the voltage at the same time;

- 2) Power consumption and load partial voltage after the thermistor (into high resistance state) and varistor operation:

- 2-1) When the power consumption of thermistor and varistor is far greater than the load power, voltage protection is applied;

- 2-2) Partial voltage protection when the power consumption of thermistor and varistor is equal to or close to the load power. The varistor turn off and not conducting when it is balanced;

- 2-3) Partial voltage protection is able to withstand most of the voltage when the power consumption of thermistor and varistor is far less than the load power. The varistor turn off and not conducting when it is balanced.

应用实例一                                         应用实例二

■ PART NUMBERING 产品编号

| WMZY1 | - | 2 | 07 | A |
| --- | --- | --- | --- | --- |

⑴       ⑵   ⑶    ⑷

⑴ Series系列：WMZY1-扁脚型的热压敏电阻 WMZY2-圆脚型的热压敏电阻

⑵ Operating Voltage 工作电压：2-220V   1-110V  5-57.7V  7★-380V

⑶ Size of Varistor 压敏电阻尺寸： 07 –Φ7  08 /10–Φ10  11/14 - Φ14   14 /20– Φ20

⑷ Serial number 同电压、同压敏不同PTC热敏电阻，产品序号

■ SPECIFICATION S TABLE规格表

☆WMZY1

| - 序号 | - 型号 | - 规格 | 热敏电阻 |  |  |  |  |  | 压敏电阻 |  | 标志 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 额定 零功率 电阻值 Rn (Ω) | 最大不动 作电流 Int70 ( mA ) | 动作 电流 It25 ( mA ) | 动作 电压 Ut （Vrms） | 居里 温度 TC (℃ ) | 芯片 尺寸 (mm) | 压敏 电压 ±10% (V) | 尺寸 (mm) |  |
| - 1 | WMZY1-206 | WMZY1-115A201V-6D391K | - 150～250 | - 25 | - 120 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф6 | - W - 206 |
| - 2 | WMZY1-207 | - WMZY1-115A201V-7D391K | - 150～250 | - 25 | - 120 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф7 | - W - 207 |
| - 4 | WMZY1-207B | - WMZY1-115A301N-7D391K | - 200～400 | - 20 | - 100 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф7 | - W - 207B |
| - 5 | WMZY1-208 | - WMZY1-115A201V-10D391K | - 150～250 | - 25 | - 120 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208 |
| - 6 | WMZY1-208A | - WMZY1-115A141N-10D391K | - 80～150 | - 30 | - 150 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208A |
| - 7 | WMZY1-208B | - WMZY1-115A301N-10D391K | - 200～400 | - 20 | - 100 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208B |
| - 8 | WMZY1-210 | - WMZY1-115B90RN-10D391K | - 60～120 | - 50 | - 250 | - 330 | - 115±7 | - Ф7*3.5 | - 390 | - Ф10 | - W - 210 |
| - 9 | WMZY1-210A | - WZMY1-73B151N-10D391K | - 100～200 | - 20 | - 150 | - 330 | - 73±7 | - Ф7*3.5 | - 390 | - Ф10 | - W - 210A |
| - 10 | WMZY1-210B | - WMZY1-115B131X-10D391K | - 100～160 | - 40 | - 200 | - 330 | - 115±7 | - Ф7*3.5 | - 390 | - Ф10 | - W - 210B |
| - 11 | WMZY1-210C | - WMZY1-115B40RV-10D391K | - 30～50 | - 60 | - 300 | - 330 | - 115±7 | - Ф7*3.5 | - 390 | - Ф10 | - W - 210C |
| - 12 | WMZY1-211 | - WMZY1-115B90RN-14D391K | - 60～120 | - 50 | - 250 | - 330 | - 115±7 | - Ф7*4.7 | - 390 | - Ф14 | - W - 211 |
| - 13 | WMZY1-211A | - WZMY1-73LB151N-10D391K | - 100～200 | - 20 | - 150 | - 330 | - 73±7 | - Ф8.0*4.7 | - 390 | - Ф14 | - W - 211A |
| - 14 | WMZY1-211B | - WMZY1-115B131X-10D391K | - 100～160 | - 40 | - 200 | - 330 | - 115±7 | - Ф7*4.7 | - 390 | - Ф14 | - W - 211B |
| - 15 | WMZY1-211C | - WMZY1-115LB40RV-10D391K | - 30～50 | - 60 | - 300 | - 330 | - 115±7 | - Ф8.0*4.7 | - 390 | - Ф14 | - W - 211C |
| - 16 | WMZY1-110 | - WMZY1-115B60RN-10D181K | - 60～80 | - 40 | - 150 | - 165 | - 115±7 | - Ф7*3.5 | - 180 | - Ф10 | - W - 110 |
| - 17 | WMZY1-111 | - WMZY1-115B60RN-14D181K | - 60～80 | - 40 | - 150 | - 165 | - 115±7 | - Ф7*4.7 | - 180 | - Ф14 | - W - 111 |
| - 18 | WMZY1-510 | - WMZY1-115B40RV-10D900K | - 30～50 | - 60 | - 300 | - 86 | - 115±7 | - Ф7*3.5 | - 90～120 | - Ф10 | - W - 510 |
| - 19 | WMZY1-511 | - WMZY1-115LB40RV-10D900K | - 30～50 | - 60 | - 300 | - 86 | - 115±7 | - Ф8.0*4.7 |  | - Ф14 | - W - 511 |
| - 20 | WMZY1-214 | - WMZY1-135M60RN-14D391K | - 40～80 | - 100 | - 400 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214 |
| - 21 | WMZY1-214A | - WMZY1-135M20RV-14D391K | - 15～25 | - 150 | - 600 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214A |
| - 22 | WMZY1-214B | - WZMY1-135M40RV-14D391K | - 30～50 | - 120 | - 480 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214B |
| - 23 | WMZY1-714 | - WMZY1-135M20RV-14D751K | - 15～25 | - 150 | - 600 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714 |
| - 24 | WMZY1-714A | - WMZY1-135M40RV-14D751K | - 30～50 | - 120 | - 480 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714A |
| - 25 | WMZY1-714B | - WMZY1-135M60RN-14D751K | - 40～80 | - 100 | - 400 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714B |
| - 26 | WMZY1-720 | - WMZY1-115SL20RV-20D751K | - 15～25 | - 150 | - 450 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720 |
| - 27 | WMZY1-720A | - WMZY1-115SL40RV-20D751K | - 30～50 | - 110 | - 400 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720A |
| - 28 | WMZY1-720B | - WMZY1-115SL60RN-20D751K | - 40～80 | - 90 | - 360 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720B |

☆WMZY2

| - 序号 | - 型号 | - 规格 | 热敏电阻 |  |  |  |  |  | 压敏电阻 |  | 标志 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 额定 零功率 电阻值 Rn (Ω) | 最大不动 作电流 Int70 ( mA ) | 动作 电流 It25 ( mA ) | 动作 电压 Ut （Vrms） | 居里 温度 TC (℃ ) | 芯片 尺寸 (mm) | 压敏 电压 ±10% (V) | 尺寸 (mm) |  |
| - 1 | WMZY2-208 | - WMZY2-115A201V-10D391K | - 150～250 | - 25 | - 120 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208 |
| - 2 | WMZY2-208A | WMZY2-115A141N-10D391K | - 80～150 | - 30 | - 150 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208A |
| - 3 | WMZY2-208B | WMZY2-115A301N-10D391K | - 200～400 | - 20 | - 100 | - 330 | - 115±7 | - Ф5*3.5 | - 390 | - Ф10 | - W - 208B |
| - 4 | WMZY2-211 | WMZY2-115B90RN-14D391K | - 60～120 | - 50 | - 250 | - 330 | - 115±7 | - Ф7*4.7 | - 390 | - Ф14 | - W - 211 |
| - 5 | WMZY2-211A | WZMY2-73LB151N-10D391K | - 100～200 | - 20 | - 150 | - 330 | - 73±7 | - Ф8.0*4.7 | - 390 | - Ф14 | - W - 211A |
| - 6 | WMZY2-211B | WMZY2-115B131X-10D391K | - 100～160 | - 40 | - 200 | - 330 | - 115±7 | - Ф7*4.7 | - 390 | - Ф14 | - W - 211B |
| - 7 | WMZY2-211C | WMZY2-115LB40RV-10D391K | - 30～50 | - 60 | - 300 | - 330 | - 115±7 | - Ф8.0*4.7 | - 390 | - Ф14 | - W - 211C |
| - 8 | WMZY2-111 | WMZY2-115B60RN-14D181K | - 60～80 | - 40 | - 150 | - 165 | - 115±7 | - Ф7*4.7 | - 180 | - Ф14 | - W - 111 |
| - 9 | WMZY1-511 | WMZY2-115LB40RV-10D900K | - 30～50 | - 60 | - 300 | - 86 | - 115±7 | - Ф8.0*4.7 | - 90~120 | - Ф14 | - W - 511 |
| - 10 | WMZY1-214 | WMZY2-135M60RN-14D391K | - 40～80 | - 100 | - 400 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214 |
| - 11 | WMZY1-214A | WMZY2-135M20RV-14D391K | - 15～25 | - 150 | - 600 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214A |
| - 12 | WMZY1-214B | WZMY2-135M40RV-14D391K | - 30～50 | - 120 | - 480 | - 330 | - 135±7 | - Ф10*4.7 | - 390 | - Ф14 | - W - 214B |
| - 13 | WMZY1-714 | WMZY2-135M20RV-14D751K | - 15～25 | - 150 | - 600 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714 |
| - 14 | WMZY1-714A | WMZY2-135M40RV-14D751K | - 30～50 | - 120 | - 480 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714A |
| - 15 | WMZY1-714B | WMZY2-135M60RN-14D751K | - 40～80 | - 100 | - 400 | - 570 | - 135±7 | - Ф10*4.7 | - 750 | - Ф14 | - W - 714B |
| - 16 | WMZY1-720 | WMZY2-115SL20RV-20D751K | - 15～25 | - 150 | - 450 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720 |
| - 17 | WMZY1-720A | WMZY2-115SL40RV-20D751K | - 30～50 | - 110 | - 400 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720A |
| - 18 | WMZY1-720B | WMZY2-115SL60RN-20D751K | - 40～80 | - 90 | - 360 | - 570 | - 115±7 | - Ф13*4.7 | - 750 | - Ф20 | - W - 720B |